⭐ COMPLETE NETWORKING FLASHCARDS (2.1–2.8)

Source-balanced: Messer + Troy + TIA

🔵 2.1 — Ports, Protocols, TCP vs UDP

Flashcard 1 — FTP

Q: What is FTP used for and what ports does it use?

A: File Transfer Protocol transfers files using TCP ports 20 (data) and 21 (control). Acronym: FTP

Real-world example: Uploading website files to a hosting server.

Hit word: Files

Memory Trick: “20 moves the data, 21 opens the door.”

Flashcard 2 — SSH

Q: What is SSH and what port does it use?

A: Secure Shell provides encrypted remote command-line access on TCP 22.

Acronym: SSH

Real-world example: Admin logging into a Linux server securely.

Hit word: Secure

Memory Trick: “22 = 2 keys → encryption.”

Flashcard 3 — Telnet

Q: What is Telnet and why is it insecure?

A: Remote CLI that sends everything in plaintext on TCP 23.

Acronym: N/A

Real-world example: Legacy router access.

Hit word: Plaintext Memory Trick: “23 = 2-3 people can read your password.”

Flashcard 4 — SMTP

Q: What is SMTP used for?

A: Sends outgoing email on TCP 25.

Acronym: SMTP

Real-world example: Gmail sending mail to Outlook servers.

Hit word: Send

Memory Trick: “25 letters in the alphabet → sending letters.”

Flashcard 5 — DNS

Q: What does DNS do?

A: Translates names ↔ IP addresses using TCP/UDP 53.

Acronym: DNS

Real-world example: Typing google.com and reaching the server.

Hit word: Phonebook

Memory Trick: “5-3 looks like S-E → Site to Endpoint.”

Flashcard 6 — DHCP

Q: What does DHCP do? A: Automatically assigns IP addresses using UDP 67/68. Acronym: DHCP Real-world example: Laptop joining Wi-Fi and getting an IP. Hit word: Automatic Memory Trick: “6-7-8 → DHCP hands out IPs in order.”

Flashcard 7 — HTTP

Q: What is HTTP used for? A: Web browsing on TCP 80. Acronym: HTTP Real-world example: Loading a non-secure website. Hit word: Web Memory Trick: “80 = 8-0 → looks like GO → websites go.”

Flashcard 8 — HTTPS

Q: What is HTTPS used for? A: Secure web browsing on TCP 443. Acronym: HTTPS Real-world example: Online banking. Hit word: Encrypted Memory Trick: “4-4-3 = For-For-Security.”

Flashcard 9 — POP3

Q: What does POP3 do? A: Downloads email from server on TCP 110. Acronym: POP Real-world example: Old Outlook clients. Hit word: Pull Memory Trick: “POP = Pull Off the server.”

Flashcard 10 — IMAP

Q: What does IMAP do? A: Syncs email across devices on TCP 143. Acronym: IMAP Real-world example: Gmail syncing on phone + laptop. Hit word: Sync Memory Trick: “1-4-3 = I Love You → IMAP loves syncing.”

Flashcard 11 — SMB

Q: What is SMB used for? A: Windows file/printer sharing on TCP 445. Acronym: SMB Real-world example: Accessing \\SERVER\SHARE. Hit word: Share Memory Trick: “4-4-5 = For-For-Files.”

Flashcard 12 — RDP

Q: What is RDP used for? A: Remote desktop GUI access on TCP 3389. Acronym: RDP Real-world example: IT remoting into a user’s PC. Hit word: Remote Memory Trick: “33 = two monitors, 89 = keyboard/mouse.”

Flashcard 13 — TCP vs UDP

Q: What is the difference between TCP and UDP? A: TCP = reliable, connection-oriented. UDP = fast, connectionless. Acronym: N/A Real-world example: TCP for banking, UDP for streaming. Hit word: Handshake vs No handshake Memory Trick: “TCP = Telephone call, UDP = Yelling across the room.”

🔵 2.2 — Wireless Networking (Wi-Fi Standards, Channels, Security)

Flashcard 14 — 802.11a

Q: What is 802.11a? A: 5 GHz, 54 Mbps, short range. Acronym: N/A Real-world example: Early enterprise APs. Hit word: 5 GHz Memory Trick: “A = Airport → fast but short runway.”

Flashcard 15 — 802.11b

Q: What is 802.11b? A: 2.4 GHz, 11 Mbps, long range. Acronym: N/A Real-world example: Old home routers. Hit word: Slow Memory Trick: “B = Basic Wi-Fi.”

Flashcard 16 — 802.11g

Q: What is 802.11g? A: 2.4 GHz, 54 Mbps, backward compatible with b. Acronym: N/A Real-world example: Mid-2000s routers. Hit word: Upgrade Memory Trick: “G = Good Wi-Fi.”

Flashcard 17 — 802.11n

Q: What is 802.11n? A: 2.4/5 GHz, up to 600 Mbps, uses MIMO. Acronym: MIMO Real-world example: Modern home routers. Hit word: Dual-band Memory Trick: “N = Next-gen.”

Flashcard 18 — 802.11ac

Q: What is 802.11ac? A: 5 GHz only, multi-gigabit, MU-MIMO. Acronym: MU-MIMO Real-world example: Wi-Fi 5 routers. Hit word: Fast Memory Trick: “AC = Air Conditioner → cool and fast.”

Flashcard 19 — 802.11ax

Q: What is 802.11ax? A: Wi-Fi 6, 2.4/5/6 GHz, high-density performance. Acronym: OFDMA Real-world example: Wi-Fi 6E routers. Hit word: Efficiency Memory Trick: “AX = Axe → cuts through congestion.”

Flashcard 20 — Channels

Q: What are the non-overlapping 2.4 GHz channels? A: 1, 6, 11 Acronym: N/A Real-world example: Fixing Wi-Fi interference. Hit word: Clean Memory Trick: “1-6-11 = 5-5-5 spacing.”

Flashcard 21 — Wi-Fi Security

Q: What are the Wi-Fi security types? A: WEP (weak), WPA, WPA2 (AES), WPA3 (strongest). Acronym: AES Real-world example: Securing a home router. Hit word: Encryption Memory Trick: “WEP = Weak, WPA2 = Works, WPA3 = Wow.”

🔵 2.3 — Networked Host Services

Flashcard 22 — DNS Server

Q: What does a DNS server do? A: Resolves names ↔ IP addresses. Acronym: DNS Real-world example: Resolving google.com. Hit word: Lookup Memory Trick: “DNS = Does Name Searches.”

Flashcard 23 — DHCP Server

Q: What does a DHCP server do? A: Assigns IP, subnet mask, gateway automatically. Acronym: DHCP Real-world example: Laptop joining Wi-Fi. Hit word: Automatic Memory Trick: “DORA → Discover, Offer, Request, Acknowledge.”

Flashcard 24 — File Server

Q: What does a file server do? A: Stores shared files for users. Acronym: N/A Real-world example: Company shared drive. Hit word: Storage Memory Trick: “File server = Filing cabinet.”

Flashcard 25 — Print Server

Q: What does a print server do? A: Manages print jobs and queues. Acronym: N/A Real-world example: Office printer spooler. Hit word: Queue Memory Trick: “Print server = Printer traffic cop.”

Flashcard 26 — Syslog Server

Q: What does a syslog server do? A: Collects logs from network devices. Acronym: N/A Real-world example: Firewall log monitoring. Hit word: Logs Memory Trick: “Syslog = System Log Storage.”

Flashcard 27 — NTP Server

Q: What does NTP do? A: Synchronizes time across devices. Acronym: NTP Real-world example: Ensuring logs match timestamps. Hit word: Time Memory Trick: “NTP = Never Time-Problem.”

🔵 2.4 — Networking Hardware

Flashcard 28 — Router

Q: What does a router do? A: Routes traffic between networks. Acronym: N/A Real-world example: Home router connecting to ISP. Hit word: Gateway Memory Trick: “Router = Road-director.”

Flashcard 29 — Switch

Q: What does a switch do? A: Connects devices in a LAN using MAC addresses. Acronym: N/A Real-world example: Office network switch. Hit word: MAC Memory Trick: “Switch = Smart plugboard.”

Flashcard 30 — Access Point

Q: What does an AP do? A: Provides wireless connectivity. Acronym: AP Real-world example: Wi-Fi hotspot. Hit word: Wireless Memory Trick: “AP = Air Point.”

🔵 2.5 — Network Types

Flashcard 31 — LAN

Q: What is a LAN? A: Local network in a building or home. Acronym: LAN Real-world example: Office network. Hit word: Local Memory Trick: “LAN = Local Area Nearby.”

Flashcard 32 — WAN

Q: What is a WAN? A: Connects networks over long distances. Acronym: WAN Real-world example: ISP backbone. Hit word: Wide Memory Trick: “WAN = World Area Network.”

🔵 2.6 — Cables & Connectors

Flashcard 33 — Cat5e

Q: What is Cat5e used for? A: 1 Gbps Ethernet. Acronym: N/A Real-world example: Home Ethernet. Hit word: Gigabit Memory Trick: “5e = 5 ‘easy’ gigabit.”

Flashcard 34 — Single-mode Fiber

Q: What is single-mode fiber? A: Long-distance laser-based fiber. Acronym: SMF Real-world example: ISP backbone. Hit word: Long Memory Trick: “Single = Straight line = Long distance.”

2.7 — IP Addressing

Flashcard 36 — APIPA

Q: What is APIPA and when is it used?

A: Automatic Private IP Addressing assigns 169.254.x.x when DHCP fails.

Acronym: APIPA

Real-world example: A laptop shows “No Internet” and has a 169.254 address.

Hit word: DHCP fail

Memory Trick: “169 = I-Need-(a)Gateway.” APIPA has no gateway, so no internet.

Flashcard 37 — Subnet Mask Purpose

Q: What does a subnet mask do?

A: Defines which part of an IP address is the network vs host portion.

Acronym: N/A

Real-world example: 255.255.255.0 for a home network.

Hit word: Divide

Memory Trick: “Mask = What part you hide (host).”

Flashcard 38 — Default Gateway

Q: What is the default gateway? A: The router address that sends traffic outside the local network. Acronym: DG Real-world example: 192.168.1.1 on home routers. Hit word: Exit Memory Trick: “Gateway = Gate to the world.”

Flashcard 39 — IPv4 vs IPv6

Q: What is the difference between IPv4 and IPv6? A: IPv4 = 32-bit, IPv6 = 128-bit with vastly more addresses. Acronym: N/A Real-world example: IPv6 used by modern ISPs. Hit word: Bigger Memory Trick: “IPv6 = 4× the digits → future-proof.”

Flashcard 40 — CIDR Notation

Q: What is CIDR notation? A: A shorthand for subnet masks using “/” (e.g., /24). Acronym: CIDR Real-world example: 192.168.1.0/24 Hit word: Slash Memory Trick: “CIDR = Cut It Down Rapidly.”

⭐ 2.8 — Network Tools

Flashcard 41 — Cable Tester

Q: What does a cable tester do? A: Verifies continuity, pin-outs, and wiring of Ethernet cables. Acronym: N/A Real-world example: Checking if a patch cable is bad. Hit word: Verify Memory Trick: “Tester = Tells the truth.”

Flashcard 42 — Loopback Plug

Q: What is a loopback plug used for? A: Tests network ports by looping signal back into the NIC. Acronym: N/A Real-world example: Diagnosing a bad Ethernet port. Hit word: Self-test Memory Trick: “Loopback = Loops back to itself.”

Flashcard 43 — Punchdown Tool

Q: What does a punchdown tool do? A: Terminates wires into patch panels or keystone jacks. Acronym: N/A Real-world example: Wiring a network closet. Hit word: Terminate Memory Trick: “Punch it down to lock it in.”

Flashcard 44 — Tone Generator & Probe

Q: What does a toner/probe do? A: Sends a tone through a cable so the probe can locate it. Acronym: N/A Real-world example: Finding a cable in a wall. Hit word: Trace Memory Trick: “Tone = Tune to the wire.”

Flashcard 45 — Multimeter

Q: What does a multimeter measure? A: Voltage, continuity, resistance. Acronym: N/A Real-world example: Checking if a power adapter works. Hit word: Measure Memory Trick: “Multi = Many tests in one.”

Flashcard 46 — Wi-Fi Analyzer

Q: What does a Wi-Fi analyzer do? A: Detects signal strength, interference, and channel usage. Acronym: N/A Real-world example: Fixing slow Wi-Fi by changing channels. Hit word: Interference Memory Trick: “Analyzer = Analyzes the air.”

Flashcard 47 — OTDR

Q: What does an OTDR do? A: Tests fiber for breaks, distance, and signal loss. Acronym: OTDR (Optical Time Domain Reflectometer) Real-world example: Finding a fiber cut in a long run. Hit word: Fiber Memory Trick: “Reflectometer = Reflects light back to report damage.”

Flashcard 48 — Patch Panel

Q: What is a patch panel used for? A: Central termination point for network cabling. Acronym: N/A Real-world example: Network closet organization. Hit word: Hub Memory Trick: “Patch panel = Cable parking lot.”

Flashcard 49 — Spectrum Analyzer

Q: What does a spectrum analyzer do? A: Measures RF interference across frequencies. Acronym: N/A Real-world example: Diagnosing Wi-Fi interference from microwaves. Hit word: RF Memory Trick: “Spectrum = See the invisible waves.”

Flashcard 50 — Cable Crimper

Q: What does a crimper do? A: Attaches RJ-45 connectors to Ethernet cable ends. Acronym: N/A Real-world example: Making custom Ethernet cables. Hit word: Attach Memory Trick: “Crimp = Clamp.”