⭐ FLASHCARDS — CompTIA A+ 220-1201 TROUBLESHOOTING (5.1–5.8)

Most Common / Most Tested Topics Only

🔵 5.1 — Motherboards, RAM, CPUs, Power

Flashcard 1 — POST Beeps

Q: What do POST beep codes indicate? A: Hardware failure before video loads (RAM, GPU, motherboard). Acronym: POST Real-world example: 3 beeps = RAM failure. Hit word: Beep = Bad hardware Memory Trick: If it beeps, it speaks.

Flashcard 2 — BSOD (Windows)

Q: What does a BSOD usually indicate? A: Faulty hardware or driver failure. Acronym: BSOD Real-world example: PAGE_FAULT_IN_NONPAGED_AREA → bad RAM. Hit word: Crash Memory Trick: Blue = Bad RAM or Bad Driver.

Flashcard 3 — Mac Pinwheel of Death

Q: What causes the Mac PWOD? A: Hung app, corrupted permissions, OS corruption. Acronym: PWOD Real-world example: Safari freezes and cursor spins forever. Hit word: Freeze Memory Trick: Spinning = System stuck.

Flashcard 4 — Blank Screen on Boot

Q: What causes a blank screen even though fans spin? A: Loose cable, bad GPU, wrong display output, low brightness. Acronym: N/A Real-world example: Laptop output set to HDMI only. Hit word: No video Memory Trick: Blank = Check basics first.

Flashcard 5 — No Power

Q: What causes a PC to show no signs of power? A: Bad PSU, bad cable, dead outlet, tripped power strip. Acronym: PSU Real-world example: PC dead after storm. Hit word: Dead Memory Trick: No lights = No juice.

Flashcard 6 — Overheating

Q: What causes overheating? A: Dust, bad fan, blocked vents, missing slot covers. Acronym: N/A Real-world example: PC shuts down after gaming. Hit word: Heat Memory Trick: Hot PC = Dust city.

Flashcard 7 — Random Shutdowns

Q: What causes random shutdowns? A: Bad PSU, bad motherboard, CPU overheating. Acronym: N/A Real-world example: PC turns off after 10 minutes. Hit word: Intermittent Memory Trick: Random = PSU or CPU.

Flashcard 8 — RAM Issues

Q: What are signs of bad RAM? A: BSOD, random reboots, POST beeps, app crashes. Acronym: RAM Real-world example: BSOD during boot. Hit word: Memory Memory Trick: Bad RAM = Bad Random behavior.

Flashcard 9 — Capacitor Swelling

Q: What does a swollen capacitor indicate? A: Failing motherboard. Acronym: N/A Real-world example: PC boots only sometimes. Hit word: Bulge Memory Trick: Bulging cap = Board’s last lap.

Flashcard 10 — CMOS Battery Failure

Q: What happens when the CMOS battery dies? A: Wrong date/time, BIOS settings reset. Acronym: CMOS Real-world example: Clock resets to 2009. Hit word: Time Memory Trick: Dead CMOS = Time loss.

🔵 5.2 — Drives & RAID

Flashcard 11 — Grinding Noise

Q: What does grinding from a hard drive mean? A: Head hitting platter → imminent failure. Acronym: N/A Real-world example: Laptop dropped while running. Hit word: Head crash Memory Trick: Grinding = Goodbye drive.

Flashcard 12 — Clicking Noise (“Click of Death”)

Q: What does clicking indicate? A: Read/write head failure. Acronym: N/A Real-world example: PC freezes then clicks repeatedly. Hit word: Click Memory Trick: Click = Critical.

Flashcard 13 — Boot Device Not Found

Q: What causes “Boot device not found”? A: Loose cable, wrong boot order, failing drive. Acronym: N/A Real-world example: BIOS can’t see SSD. Hit word: Boot Memory Trick: No boot = No drive.

Flashcard 14 — OS Not Found

Q: What causes “Operating system not found”? A: Corrupt MBR, missing active partition. Acronym: MBR Real-world example: Windows won’t load after crash. Hit word: MBR Memory Trick: OS missing = MBR kissing goodbye.

Flashcard 15 — SMART Failure

Q: What does a SMART warning mean? A: Drive predicts imminent failure. Acronym: SMART Real-world example: “Backup immediately” message. Hit word: Fail Memory Trick: SMART = Save My A* Right-now.*

Flashcard 16 — RAID Degraded

Q: What causes a RAID array to degrade? A: One drive failed or disconnected. Acronym: RAID Real-world example: RAID 1 mirror shows “Degraded.” Hit word: Mirror Memory Trick: RAID down = Drive down.

Flashcard 17 — Slow Read/Write

Q: What causes slow disk performance? A: Fragmentation, low RAM (paging), failing drive. Acronym: N/A Real-world example: Apps take forever to open. Hit word: Slow Memory Trick: Slow disk = Slow system.

🔵 5.3 — Video, Projector, Display

Flashcard 18 — No Display

Q: What causes “No signal”? A: Wrong input, loose cable, bad GPU. Acronym: N/A Real-world example: HDMI plugged into wrong port. Hit word: Signal Memory Trick: No signal = No connection.

Flashcard 19 — Dim Display

Q: What causes a dim laptop screen? A: Bad inverter, bad backlight, low brightness. Acronym: N/A Real-world example: Screen barely visible. Hit word: Dim Memory Trick: Dim = Inverter dimming.

Flashcard 20 — Flickering Screen

Q: What causes flickering? A: Loose cable, bad GPU, wrong refresh rate. Acronym: N/A Real-world example: Monitor flickers at 30 Hz. Hit word: Flicker Memory Trick: Flicker = Fix cable first.

Flashcard 21 — Artifacts

Q: What causes artifacts (lines, blocks)? A: Overheating GPU or bad VRAM. Acronym: VRAM Real-world example: Lines appear during gaming. Hit word: Artifacts Memory Trick: Artifacts = GPU too hot.

Flashcard 22 — Projector Dim Image

Q: What causes a dim projector image? A: Lamp near end of life, dusty optics. Acronym: N/A Real-world example: Classroom projector fading. Hit word: Lamp Memory Trick: Dim projector = Dying lamp.

🔵 5.4 — Mobile Devices

Flashcard 23 — Touchscreen Not Responding

Q: What causes touchscreen failure? A: Bad digitizer, OS freeze. Acronym: N/A Real-world example: Phone won’t register taps. Hit word: Touch Memory Trick: No touch = Digitizer dead.

Flashcard 24 — Rapid Battery Drain

Q: What causes fast battery drain? A: Background apps, old battery, overheating. Acronym: N/A Real-world example: Phone dies in 2 hours. Hit word: Drain Memory Trick: Hot phone = Hungry battery.

Flashcard 25 — No Wi-Fi

Q: What causes no Wi-Fi? A: Airplane mode, wrong password, router issue. Acronym: N/A Real-world example: Phone won’t connect at home. Hit word: Wi-Fi Memory Trick: No Wi-Fi = Check airplane first.

Flashcard 26 — Swollen Battery

Q: What does a swollen battery indicate? A: Chemical failure → dangerous. Acronym: N/A Real-world example: Phone back cover lifting. Hit word: Swollen Memory Trick: Swollen = Stop using.

🔵 5.5 — Printers

Flashcard 27 — Laser Printer Streaks

Q: What causes streaks in laser prints? A: Dirty drum or corona wire. Acronym: N/A Real-world example: Vertical lines on printouts. Hit word: Streaks Memory Trick: Streaks = Dirty drum.

Flashcard 28 — Ghosting

Q: What causes ghosting? A: Bad fuser or low fuser temperature. Acronym: N/A Real-world example: Faint duplicate images. Hit word: Ghost Memory Trick: Ghost = Fuser toast.

Flashcard 29 — Inkjet Streaks

Q: What causes inkjet streaks? A: Clogged nozzles. Acronym: N/A Real-world example: Missing lines in photos. Hit word: Nozzle Memory Trick: Streaks = Clean nozzles.

Flashcard 30 — Printer Offline

Q: What causes “Printer offline”? A: Wrong driver, wrong IP, cable unplugged. Acronym: N/A Real-world example: Office printer unreachable. Hit word: Offline Memory Trick: Offline = IP or driver.

🔵 5.6 — Networks

Flashcard 31 — APIPA (169.254.x.x)

Q: What does an APIPA address mean? A: DHCP server unreachable. Acronym: APIPA Real-world example: PC can’t get IP. Hit word: DHCP Memory Trick: 169 = No DHCP.

Flashcard 32 — Slow Network

Q: What causes slow network speeds? A: Interference, bad cable, overloaded AP. Acronym: N/A Real-world example: Wi-Fi slow in crowded café. Hit word: Slow Memory Trick: Slow = Signal or cable.

Flashcard 33 — IP Conflict

Q: What causes an IP conflict? A: Two devices using same IP. Acronym: IP Real-world example: Printer and PC share IP. Hit word: Conflict Memory Trick: Two IPs = One problem.

🔵 5.7 — Security Issues

Flashcard 34 — Pop-ups & Redirects

Q: What causes pop-ups and browser redirects? A: Malware or adware. Acronym: N/A Real-world example: Browser opens random sites. Hit word: Malware Memory Trick: Pop-ups = Infection.

Flashcard 35 — Unauthorized Access

Q: What causes unauthorized logins? A: Weak passwords, stolen credentials. Acronym: N/A Real-world example: Account accessed from unknown location. Hit word: Access Memory Trick: Weak password = Wide open.

🔵 5.8 — Common Issues

Flashcard 36 — Slow Performance

Q: What causes slow performance? A: Low RAM, malware, overheating, full drive. Acronym: N/A Real-world example: PC takes minutes to open apps. Hit word: Slow Memory Trick: Slow = RAM or malware.

Flashcard 37 — System Freezes

Q: What causes freezing? A: Overheating, bad RAM, failing drive. Acronym: N/A Real-world example: PC locks up randomly. Hit word: Freeze Memory Trick: Freeze = Faulty hardware.

Flashcard 38 — Boot Loop

Q: What causes a boot loop? A: Corrupt OS, bad driver, failing hardware. Acronym: N/A Real-world example: Windows restarts endlessly. Hit word: Loop Memory Trick: Loop = Load fails.